# 759 switch settings

# CORE VCC SELECT

|  | 3.2V | 3.1V | 3.0V | 2.9V | 2.8V | 2.7V | 2.6V | 2.5V | 2.4V | 2.3V | 2.2V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SW1-6 | ON | ON | ON | ON | ON | OFF | OFF | OFF | OFF | OFF | OFF |
| SW1-5 | OFF | OFF | OFF | OFF | OFF | ON | ON | ON | ON | ON | ON |
| SW1-4 | ON | OFF | OFF | OFF | OFF | ON | ON | ON | ON | OFF | OFF |
| SW1-3 | OFF | ON | ON | OFF | OFF | ON | ON | OFF | OFF | ON | ON |
| SW1-2 | OFF | ON | OFF | ON | OFF | ON | OFF | ON | OFF | ON | OFF |
| SW1-1 | OFF | OFF | OFF | OFF | OFF | OFF | OFF | OFF | OFF | OFF | OFF |

|  | 2.1V | 2.0V | 1.9V | 1.85V | 1.8V | 1.75V | 1.7V | 1.65V | 1.6V | 1.55V | 1.5V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SW1-6 | OFF | OFF | OFF | OFF | OFF | OFF | OFF | OFF | OFF | OFF | OFF |
| SW1-5 | ON | ON | OFF | OFF | OFF | OFF | OFF | OFF | OFF | OFF | OFF |
| SW1-4 | OFF | OFF | ON | ON | ON | ON | ON | ON | ON | OFF | OFF |
| SW1-3 | OFF | OFF | ON | ON | ON | OFF | OFF | OFF | OFF | ON | ON |
| SW1-2 | ON | OFF | ON | OFF | OFF | ON | ON | OFF | OFF | ON | ON |
| SW1-1 | OFF | OFF | OFF | ON | OFF | ON | OFF | ON | OFF | ON | OFF |

# IO VCC SELECT

|  | SW1-7 | SW1-8 |
| --- | --- | --- |
| 2.5V | ON | ON |
| 3.3V | OFF | OFF |

# BASE FREQUENCY SELECT

|  | SW2-4 |
| --- | --- |
| 60MHz | ON |
| 66MHz | OFF |

# AMD CPU SETTING

|  | SW2-5 | SW2-6 | SW2-7 | SW2-8 |
| --- | --- | --- | --- | --- |
| 5.5X | OFF | ON | OFF | OFF |
| 4.0X | OFF | ON | OFF | ON |
| 5.0X | OFF | ON | ON | OFF |
| 4.5X | OFF | ON | ON | ON |
| 3.5X | OFF | OFF | OFF | OFF |
| 2.0X | OFF | OFF | OFF | ON |
| 3.0X | OFF | OFF | ON | OFF |
| 2.5X | OFF | OFF | ON | ON |

# INTEL CPU SETTING

|  | SW2-5 | SW2-6 | SW2-7 | SW2-8 |
| --- | --- | --- | --- | --- |
| 3.5X | ON | OFF | OFF | OFF |
| 4.0X | OFF | OFF | ON | ON |
| 2.0X | ON | OFF | OFF | ON |
| 3.0X | ON | OFF | ON | OFF |
| 2.5X | ON | OFF | ON | ON |